MC2010

Unit: mm

MC4010

Unit: mm

- A combination of multiple tracking algorithms enables tracking the maximum power point quickly and

accurately

- Innovative Max Power Point Tracking(MPPT) technology, tracking efficiency >99.9%

- Full digital technology, high charge conversion efficiency up to 98%

- LCD display design, read operating data and working condition easily

- Real-time energy statistics function

- 12/24V or 48V automatic recognition

- Flexible System battery selection: Liquid, Gel, AGM and Lithium

- Extends battery life through accurate remote temperature sensor

- Controller is protected against over-temperature due to built-in power reduction function

- Four stages battery charging process: MPPT, boost, equalization, float

- Dual automatic protection to avoid exceeding the rated charging power and current

- Multiple load control modes: Always on, Street lamp, User-defined mode

- IoT wireless communication or Bluetooth (internal or external) communication functions optional

- Optional APP version for Bluetooth communication

- With the wireless communication function of the IoT, the controller can be connected remotely through IoT/GPRS

- Monthly charging data can be calculated and displayed by grouping and graphs

- Based RS-485 standard Modbus protocol with RJ11 interface to maximize the communication needs of different occasions

- Perfect Heat dissipation design

- Full automatic electronic protect function for increased charge controller availability

|  | Item | MC2010 | MC4010 | MC6015 |
| --- | --- | --- | --- | --- |
| Battery Parameters | System Voltage | 12V/24V automatical recognization |  | 12V/24V/48V automatical recognization |
|  | Max Charging Current | 20A | 40A | 60A |
|  | MPPT Charging Voltage | before boost or equal charging stage |  |  |
|  | Boost Voltage@25℃ | 14.0~14.8V/28.0~29.6V @25℃(default:14.5/29V) |  | 14.0~14.8V/28.0~29.6V/56.0~59.2V(default :14.5/29.0/58.0V) |
|  | Equal Voltage@25℃ | 14.0~15.0V/28.0~30.0V @25℃(default:14.8/29.6V)(Liquid, AGM) |  | 14.0~15.0V/28.0~30.0V/56.0~60.0V (default:14.8/29.6/59.2V) |
|  | Float Voltage@25℃ | 13.0~14.5V/26.0~29.0V @25℃(default:13.7/27.4V) |  | 13.0~14.5V/26.0~29.0V /52.0~58.0V(default:13.7/27.4/54.8V) |
|  | Low Voltage Disconnect | 10.8~11.8V/21.6~23.6V(default:11.2/22.4V) |  | 10.8~11.8V/21.6~23.6V/43.2~47.2V(default :11.2/22.4/44.8V) |
|  | Reconnect Voltage | 11.4~12.8V/22.8~25.6V(default:12.0/24.0V) |  | 11.4~12.8V/22.8~25.6V/45.6~51.2V(default :12.0/24.0/48.0V) |
|  | Overcharge Protect | 15.8V/31.3V |  | 15.8/31.3/62.3V |
|  | Charging target voltage | 10.0~32.0V(Lithium, default: 14.4V) |  | 10.0~64.0V(Lithium, default: 29.4V) |
|  | Charging recovery voltage | 9.2~31.8V(Lithium, default: 14.0V) |  | 9.2~63.8V(Lithium, default: 28.7V) |
|  | Low voltage disconnect | 9.0~30.0V(Lithium, default: 10.6V) |  | 9.0~60.0V(Lithium, default: 21.0V) |
|  | Low voltage reconnect | 9.6~31.0V(Lithium, default: 12.0V) |  | 9.6~62.0V(Lithium, default: 22.4V) |
|  | Temperature Compensation | 35V |  | 65V |
|  | Battery Type | -4.17mV/K per cell (Boost, Equal), -3.33mV/K per cell (Float) |  |  |
|  | Max Voltage on Battery Terminal | Liquid, GEL, AGM, Lithium(default: GEL) |  |  |
| Panel Parameters | Max Voltage on PV Terminal(-20℃) *1 | 100V |  | 150V |
|  | Max Voltage on PV Terminal(25℃) | 90V |  | 138V |
|  | Max Input Power | 260W/520W | 520W/1040W | 750/1500/3000W |
|  | Dusk/Dawn detect voltage | 8.0V/16.0V |  | 8.0V/16.0V/32.0V |
|  | MPPT Tracking Range | (Battery Voltage+1V) ~ Voc*0.9 *2 |  |  |
| Load Parameters | Output Current | 20A | 30A |  |
|  | Work mode | Always on, Street lamp, User-defined mode |  |  |
| System Parameters | Max tracking efficiency | >99.9% |  |  |
|  | Max charge conversion | 98% |  |  |
|  | Dimensions | 136.6*136.6*67.1mm | 196.5*136.6*97.1mm | 262.5*186.5*97.5mm |
|  | Weight | 830g | 1300g | 2500g |
|  | Self consumption | ≤8mA (12V); ≤12mA (24V) |  | ≤16mA (12V); ≤12mA (24V)；≤10mA (48V) |
|  | Communication | RS485 (RJ11 interface) |  |  |
|  | Optional | IoT、Cyber-BT |  |  |
|  | Grounding | Common Negative |  |  |
|  | Power ternimals | 6AWG (16mm²) |  |  |
|  | Ambient temperature | -20 ~ +55℃ |  |  |
|  | Storage temperature | -25 ~ +80℃ |  |  |
|  | Ambient humidity | 0~100%RH |  |  |
|  | Protection degree | IP32 |  |  |
|  | Max Altitude | 4000m |  |  |

*1. Maximum solar panel voltage at minimum ambient operating temperature.

*2. Voc: PV-Module open circuit voltage.

*3. Slash separate values for 12V/24V/48V nominal system voltage.

|  | Item | MC2010-BT | MC4010-BT | MC6015-BT |
| --- | --- | --- | --- | --- |
| Battery Parameters | System Voltage | 12V/24V automatical recognization |  | 12V/24V/48V automatical recognization |
|  | Max Charging Current | 20A | 40A | 60A |
|  | MPPT Charging Voltage | before boost or equal charging stage |  |  |
|  | Boost Voltage@25℃ | 14.0~14.8V/28.0~29.6V @25℃(default:14.5/29V) |  | 14.0~14.8V/28.0~29.6V/56.0~59.2V(default :14.5/29.0/58.0V) |
|  | Equal Voltage@25℃ | 14.0~15.0V/28.0~30.0V @25℃(default:14.8/29.6V)(Liquid, AGM) |  | 14.0~15.0V/28.0~30.0V/56.0~60.0V (default:14.8/29.6/59.2V) |
|  | Float Voltage@25℃ | 13.0~14.5V/26.0~29.0V @25℃(default:13.7/27.4V) |  | 13.0~14.5V/26.0~29.0V /52.0~58.0V(default:13.7/27.4/54.8V) |
|  | Low Voltage Disconnect | 10.8~11.8V/21.6~23.6V(default:11.2/22.4V) |  | 10.8~11.8V/21.6~23.6V/43.2~47.2V(default :11.2/22.4/44.8V) |
|  | Reconnect Voltage | 11.4~12.8V/22.8~25.6V(default:12.0/24.0V) |  | 11.4~12.8V/22.8~25.6V/45.6~51.2V(default :12.0/24.0/48.0V) |
|  | Overcharge Protect | 15.8V/31.3V |  | 15.8/31.3/62.3V |
|  | Charging target voltage | 10.0~32.0V(Lithium, default: 14.4V) |  | 10.0~64.0V(Lithium, default: 29.4V) |
|  | Charging recovery voltage | 9.2~31.8V(Lithium, default: 14.0V) |  | 9.2~63.8V(Lithium, default: 28.7V) |
|  | Low voltage disconnect | 9.0~30.0V(Lithium, default: 10.6V) |  | 9.0~60.0V(Lithium, default: 21.0V) |
|  | Low voltage reconnect | 9.6~31.0V(Lithium, default: 12.0V) |  | 9.6~62.0V(Lithium, default: 22.4V) |
|  | Temperature Compensation | 35V |  | 65V |
|  | Battery Type | -4.17mV/K per cell (Boost, Equal), -3.33mV/K per cell (Float) |  |  |
|  | Max Voltage on Battery Terminal | Liquid, GEL, AGM, Lithium(default: GEL) |  |  |
| Panel Parameters | Max Voltage on PV Terminal(-20℃) *1 | 100V |  | 150V |
|  | Max Voltage on PV Terminal(25℃) | 90V |  | 138V |
|  | Max Input Power | 260W/520W | 520W/1040W | 750/1500/3000W |
|  | Dusk/Dawn detect voltage | 8.0V/16.0V |  | 8.0V/16.0V/32.0V |
|  | MPPT Tracking Range | (Battery Voltage+1V) ~ Voc*0.9 *2 |  |  |
| Load Parameters | Output Current | 20A | 30A |  |
|  | Work mode | Always on, Street lamp, User-defined mode |  |  |
| System Parameters | Max tracking efficiency | >99.9% |  |  |
|  | Max charge conversion | 98% |  |  |
|  | Dimensions | 136.6*136.6*67.1mm | 196.5*136.6*97.1mm | 262.5*186.5*97.5mm |
|  | Weight | 830g | 1300g | 2500g |
|  | Self consumption | ≤8mA (12V); ≤12mA (24V) |  | ≤16mA (12V); ≤12mA (24V)；≤10mA (48V) |
|  | Communication | BlueTooth, RS485 (RJ11 interface) |  |  |
|  | Optional | IoT |  |  |
|  | Grounding | Common Negative |  |  |
|  | Power ternimals | 6AWG (16mm²) |  |  |
|  | Ambient temperature | -20 ~ +55℃ |  |  |
|  | Storage temperature | -25 ~ +80℃ |  |  |
|  | Ambient humidity | 0~100%RH |  |  |
|  | Protection degree | IP32 |  |  |
|  | Max Altitude | 4000m |  |  |

*1. Maximum solar panel voltage at minimum ambient operating temperature.

*2. Voc: PV-Module open circuit voltage.

*3. Slash separate values for 12V/24V/48V nominal system vol